# STM32 Clock Tree and Clock Configuration

# Learning Outcomes

- Explain the STM32 clock tree architecture.

- Describe the purpose of various clock sources in STM32.

- Explain the importance of clock configuration in embedded systems.

- Discuss the functions of HSI, HSE, PLL, LSI, and LSE.

- Configure peripheral clock prescalers.

- Analyze the effects of incorrect clock settings on processor performance, communication peripherals, and power consumption.

- Understand how STM32CubeMX is used for clock configuration.

# 1. Introduction to Clock in Embedded Systems

Every digital system requires a clock signal to synchronize the execution of instructions and peripheral operations.

A clock is a periodic square-wave signal that determines when operations occur inside a microcontroller.

Without a clock:

- CPU cannot execute instructions.

- Timers cannot count.

- UART cannot transmit data.

- ADC conversions cannot start.

- SPI and I2C communication will fail.

Thus, the clock acts as the heartbeat of the microcontroller.

# Why is Clock Required?

The clock synchronizes:

- CPU instruction execution

- Memory access

- Peripheral operation

- Communication protocols

- Timer counting

- ADC sampling

- PWM generation

Without synchronization, hardware modules would operate unpredictably.

# STM32 Clock Sources

STM32 provides multiple clock sources to satisfy different applications.

| Clock Source | Typical Frequency | Purpose |
| --- | --- | --- |
| HSI | 16 MHz (STM32F4) / 64 MHz (STM32H7) | Internal RC oscillator |
| HSE | 4–26 MHz External Crystal/Oscillator | Accurate system clock |
| LSI | ~32 kHz | Independent watchdog, RTC |
| LSE | 32.768 kHz Crystal | Real-Time Clock |
| PLL | Variable | Frequency multiplication |

# STM32 Clock Tree

## Block Diagram

STM32 CLOCK TREE

+--------------------+

|   HSI Oscillator   |

| (High Speed Internal)

+--------------------+

|

|

+--------------------+

|   HSE Oscillator   |

| (High Speed External)

+--------------------+

|

|

+--------------------+

|   PLL (Multiplier) |

+--------------------+

|

System Clock (SYSCLK)

|

-----------------------------------------

|                  |                    |

AHB Bus          APB1 Bus            APB2 Bus

(HCLK)          (PCLK1)             (PCLK2)

|                  |                    |

|                  |                    |

CPU Core           Timers              USART1

SRAM              USART2               SPI1

Flash               SPI2                ADC

DMA                I2C                 TIM1

Understanding the Clock Tree

The STM32 clock tree distributes clock signals to every subsystem.

Flow of clock

Clock Source

↓

PLL (optional)

↓

SYSCLK

↓

AHB Prescaler

↓

HCLK

↓

APB Prescalers

↓

Peripheral Clocks

# Major Clock Sources

## 1. HSI

### HSI

High-Speed Internal Oscillator

Characteristics

- Internal RC oscillator

- No external crystal required

- Available immediately after reset

- Moderate accuracy

- Low cost

Typical Frequency

STM32F4

16 MHz

STM32H7

64 MHz

Advantages

- Fast startup

- No external hardware

- Lower cost

Disadvantages

- Less accurate

- Frequency varies with

- Temperature

- Voltage

- Manufacturing process

Applications

- Basic embedded systems

- LED blinking

- Internal testing

- Battery-powered devices

# 2. HSE

### HSE

High-Speed External Oscillator

Uses

- Crystal oscillator

- Ceramic resonator

- External clock source

Typical Frequency

4 MHz

8 MHz

16 MHz

25 MHz

Advantages

- High accuracy

- Stable frequency

- Better communication timing

Applications

- USB

- Ethernet

- CAN

- High-speed UART

- Precision timers

# 3. PLL

### PLL

Phase-Locked Loop

Purpose

The PLL multiplies the input clock frequency to generate a faster system clock.

Example

HSE = 8 MHz

PLL Multiplier = ×20

SYSCLK = 160 MHz

Benefits

- Higher CPU performance

- Flexible clock generation

- Supports different operating frequencies

# 4. LSI

### LSI

Low-Speed Internal Oscillator

Frequency

Approximately

32 kHz

Applications

- Independent Watchdog

- Low-power operation

Advantages

- No external components

Disadvantages

- Low accuracy

# 5. LSE

### LSE

Low-Speed External Oscillator

Frequency

32.768 kHz

Applications

- RTC (Real-Time Clock)

Advantages

Very high timing accuracy

Commonly used in

- Digital clocks

- Data loggers

- Calendar systems

# STM32 Clock Distribution

Clock Source

↓

PLL

↓

SYSCLK

↓

AHB Prescaler

↓

HCLK

↓

CPU

Flash

DMA

SRAM

↓

APB1 Prescaler

↓

USART2

USART3

TIM2

TIM3

I2C

↓

APB2 Prescaler

↓

USART1

ADC

TIM1

SPI1

# System Clock (SYSCLK)

SYSCLK is the main operating clock of the STM32 microcontroller.

It controls

- CPU speed

- Bus frequency

- Peripheral clocks

Clock sources for SYSCLK

- HSI

- HSE

- PLL Output

# AHB Clock (HCLK)

### AHB

Advanced High-performance Bus

Used for

- CPU

- SRAM

- Flash

- DMA

Formula

HCLK

=

SYSCLK

÷

AHB Prescaler

Example

SYSCLK

160 MHz

AHB Prescaler

2

Result

HCLK

80 MHz

# APB Clock

### APB

Advanced Peripheral Bus

Two buses

APB1

Low-speed peripherals

APB2

High-speed peripherals

Formula

PCLK

=

HCLK

÷

APB Prescaler

# Peripheral Clock Prescalers

A prescaler divides the clock frequency before supplying it to a peripheral.

Example

HCLK

160 MHz

APB Prescaler

4

Peripheral Clock

40 MHz

# Why Prescalers are Required?

Reasons

- Different peripherals have different maximum clock limits.

- Reduce power consumption.

- Maintain communication timing.

- Prevent peripheral damage.

# Importance of Clock Configuration

Proper clock configuration directly affects

- Processor speed

- Timer accuracy

- UART baud rate

- SPI clock

- ADC sampling

- PWM frequency

- USB communication

- Ethernet communication

- CAN timing

- Power consumption

# Effect on Processor Performance

Example

CPU Clock

50 MHz

Instruction

1 Clock Cycle

Execution Time

20 ns

Now increase

CPU Clock

200 MHz

Execution Time

5 ns

Higher clock frequency means

- Faster execution

- Higher throughput

- Better multitasking

- Faster interrupt handling

# Effect on Peripheral Clock

Every peripheral depends on clock frequency.

Examples

UART

Incorrect clock

↓

Wrong baud rate

↓

Communication failure

SPI

Wrong clock

↓

Data corruption

ADC

Incorrect clock

↓

Incorrect sampling

PWM

Incorrect timer clock

↓

Wrong frequency

# Effect on Communication Interfaces

## UART

Suppose

Desired baud rate

115200 bps

Clock configured incorrectly

↓

Actual baud rate becomes

98000 bps

Result

- Framing errors

- Communication failure

- Data loss

## SPI

Clock too high

↓

Slave cannot receive data

↓

Corrupted communication

Clock too low

↓

Reduced throughput

## I²C

Wrong clock

↓

Incorrect SCL frequency

↓

ACK failures

↓

Bus timeout

## USB

USB requires a precise 48 MHz clock.

If PLL is configured incorrectly

- USB enumeration fails.

- Host cannot recognize the device.

- Data transfer errors occur.

# Effect on Timers

Timer frequency

Timer Clock

=

Peripheral Clock

÷

Prescaler

Incorrect timer clock leads to

- Wrong PWM frequency

- Incorrect delays

- Servo malfunction

- Motor speed errors

# Effect on ADC

ADC conversion speed depends on the ADC clock.

Clock too fast

- Reduced conversion accuracy

- Higher noise

- Possible specification violations

Clock too slow

- Low sampling rate

- Missed fast-changing signals

# Effect on Power Consumption

Power consumption is approximately proportional to operating frequency and voltage:

[
P \propto C \times V^2 \times f
]

Where:

- P = Power

- C = Switching capacitance

- V = Supply voltage

- f = Clock frequency

Higher clock frequency

- Higher CPU power

- More heat

- Shorter battery life

Lower clock frequency

- Lower power

- Longer battery life

- Reduced performance

# STM32CubeMX Clock Configuration

STM32CubeMX provides a graphical interface to configure clocks.

Typical steps:

- Select the MCU (e.g., STM32F401RE or STM32H743ZI).

- Open the Clock Configuration tab.

- Choose the clock source (HSI or HSE).

- Configure the PLL multipliers/dividers.

- Set the desired SYSCLK frequency.

- Configure AHB, APB1, and APB2 prescalers.

- Verify that all frequencies remain within device specifications.

- Generate the initialization code.

CubeMX automatically highlights invalid clock configurations, helping prevent configuration errors.

# Practical Example

Given:

HSE = 8 MHz

PLL Multiplier = ×20

SYSCLK = 160 MHz

AHB Prescaler = 1

APB1 Prescaler = 4

APB2 Prescaler = 2

Clock calculations:

SYSCLK = 160 MHz

HCLK = 160 MHz

PCLK1 = 40 MHz

PCLK2 = 80 MHz

Peripherals on APB1 receive 40 MHz, while peripherals on APB2 receive 80 MHz.

# Advantages of Proper Clock Configuration

- Maximum CPU performance

- Accurate communication interfaces

- Stable timer operation

- Correct ADC sampling

- Reliable USB and Ethernet operation

- Lower power consumption

- Improved system stability

- Longer battery life

- Better real-time performance

# Summary

- The STM32 clock tree distributes clocks from multiple sources to the CPU and peripherals.

- HSI provides a low-cost internal clock, while HSE offers high accuracy using an external crystal.

- The PLL increases clock frequency to achieve higher system performance.

- AHB and APB prescalers divide the system clock to meet peripheral timing requirements.

- Incorrect clock configuration can cause processor slowdowns, communication failures, inaccurate timers, ADC errors, and increased power consumption.

- Proper clock configuration is essential for achieving high performance, low power, and reliable operation in embedded systems.

# Important Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| STM32 | STMicroelectronics 32-bit Microcontroller Family |
| MCU | Microcontroller Unit |
| CPU | Central Processing Unit |
| SYSCLK | System Clock |
| HSI | High-Speed Internal Oscillator |
| HSE | High-Speed External Oscillator |
| LSI | Low-Speed Internal Oscillator |
| LSE | Low-Speed External Oscillator |
| PLL | Phase-Locked Loop |
| AHB | Advanced High-performance Bus |
| APB | Advanced Peripheral Bus |
| HCLK | AHB Clock |
| PCLK1 | Peripheral Clock for APB1 |
| PCLK2 | Peripheral Clock for APB2 |
| RTC | Real-Time Clock |
| UART | Universal Asynchronous Receiver/Transmitter |
| USART | Universal Synchronous/Asynchronous Receiver/Transmitter |
| SPI | Serial Peripheral Interface |
| I²C | Inter-Integrated Circuit |
| CAN | Controller Area Network |
| USB | Universal Serial Bus |
| ADC | Analog-to-Digital Converter |
| PWM | Pulse Width Modulation |
| DMA | Direct Memory Access |
| RCC | Reset and Clock Control |
| RC | Resistor-Capacitor Oscillator |
| bps | Bits Per Second |