# STM32 System Tick (SysTick) Timer and STM32 Boot Sequence

# Learning Outcomes

- Explain the System Tick (SysTick) Timer in ARM Cortex-M processors.

- Describe the architecture and working principle of the SysTick timer.

- Explain the role of the SysTick timer in Real-Time Operating Systems (RTOS) and embedded applications.

- Configure the SysTick timer to generate periodic interrupts.

- Explain the complete STM32 boot sequence from Power-On Reset (POR) to the execution of main().

- Describe the role of the Reset Handler, Startup File, Vector Table, Data Initialization, and BSS Initialization.

# PART – I : System Tick (SysTick) Timer

# 1. Introduction

The SysTick timer is a 24-bit down-counter timer integrated into every ARM Cortex-M processor.

Unlike general-purpose timers, SysTick is part of the processor core and is available in all Cortex-M devices.

It is primarily used for:

- Periodic interrupts

- Time delays

- Task scheduling

- RTOS system tick generation

- Software timers

- Time measurement

# What is a Timer?

A timer is a hardware module that counts clock pulses to measure time intervals.

Example:

If the clock frequency is 1 MHz, one clock pulse occurs every 1 µs.

The timer counts these pulses to generate accurate delays or interrupts.

# What is SysTick?

SysTick is a dedicated timer located inside the ARM Cortex-M core.

Features:

- 24-bit countdown timer

- Auto reload capability

- Generates periodic interrupts

- Works with CPU clock or external reference clock

- Available in all Cortex-M processors

# SysTick Block Diagram

ARM Cortex-M Core

+----------------------------+

|        CPU Core            |

+----------------------------+

|

|

Processor Clock

|

+---------------+

| SysTick Timer |

| 24-bit Counter|

+---------------+

|

Reload Register (LOAD)

|

Current Value Register

|

Control Register

|

SysTick Interrupt

|

NVIC

|

CPU ISR

# SysTick Registers

The SysTick timer consists of four important registers.

| Register | Purpose |
| --- | --- |
| CTRL | Control and Status Register |
| LOAD | Reload Value Register |
| VAL | Current Counter Value |
| CALIB | Calibration Register |

# 1. CTRL Register

Controls the operation of SysTick.

Functions

- Enable timer

- Enable interrupt

- Select clock source

- Monitor COUNTFLAG

Important Bits

| Bit | Function |
| --- | --- |
| ENABLE | Starts SysTick |
| TICKINT | Enables interrupt |
| CLKSOURCE | Selects clock source |
| COUNTFLAG | Indicates timer reached zero |

# 2. LOAD Register

Stores the reload value.

Maximum value

224 − 1

= 16,777,215

After reaching zero,

The timer automatically reloads this value.

# 3. VAL Register

Contains the current counter value.

The counter decreases continuously.

Example

1000

↓

999

↓

998

↓

...

↓

0

# 4. CALIB Register

Stores factory calibration information.

Used for

- Delay generation

- Clock calibration

# Working Principle of SysTick

CPU Clock

↓

SysTick Counter

↓

Counts Down

↓

Zero Reached

↓

Interrupt Generated

↓

Counter Reloaded

↓

Process Repeats

# Role of SysTick Timer in ARM Cortex-M

The SysTick timer provides a fixed periodic interrupt used by the operating system or application.

Major roles

- Generates precise time base

- Creates periodic interrupts

- Schedules RTOS tasks

- Generates software delays

- Measures elapsed time

# Applications of SysTick

## 1. Operating Systems

Example

FreeRTOS

SysTick generates

1 ms Tick

↓

Task Scheduler

↓

Task Switching

## 2. LED Blinking

Example

Generate interrupt every

500 ms

LED toggles inside ISR.

## 3. Software Delay

Instead of busy waiting,

SysTick counts time accurately.

## 4. Periodic Sensor Sampling

Example

Read

Temperature sensor

every

100 ms

## 5. Real-Time Task Scheduling

Different tasks execute periodically.

1 ms

↓

Read Sensor

10 ms

↓

Update Display

100 ms

↓

Send UART Data

## 6. Time Measurement

Measure execution time

Example

Start SysTick

↓

Execute Function

↓

Stop SysTick

↓

Elapsed Time

# Steps to Configure SysTick

Assume

CPU Clock

16 MHz

Need

1 ms Interrupt

## Step 1

Choose clock source

Usually

Processor Clock

16 MHz

## Step 2

Calculate Reload Value

Formula

Reload

=

Clock Frequency × Desired Time −1

Example

16 MHz × 1 ms

=

16000

Reload

=

15999

## Step 3

Load Reload Register

SysTick->LOAD = 15999;

## Step 4

Clear Current Value

SysTick->VAL = 0;

## Step 5

Enable Interrupt

TICKINT = 1

## Step 6

Enable Timer

ENABLE = 1

## Step 7

Write Interrupt Service Routine

SysTick_Handler()

{

// User Code

}

# SysTick Configuration Flow

Enable Clock

↓

Calculate Reload Value

↓

Load LOAD Register

↓

Clear VAL Register

↓

Enable Interrupt

↓

Start Counter

↓

Periodic Interrupt Generated

# Advantages of SysTick

- Built into Cortex-M core

- Easy to configure

- High timing accuracy

- Supports RTOS

- Low CPU overhead

- Auto reload

- Standard across all Cortex-M processors

# Limitations

- Only one SysTick timer per processor

- Limited to 24-bit counter

- Not suitable for multiple independent timing channels

- General-purpose timers are preferred for PWM and input capture

# STM32 Boot Sequence

# Introduction

The boot sequence is the sequence of operations that occur after the STM32 microcontroller is powered on or reset, until the user application (main()) begins execution.

# What is Booting?

Booting is the process of

Power Applied

↓

CPU Starts

↓

Memory Initialized

↓

Program Starts

↓

main()

# STM32 Boot Sequence Overview

Power ON / Reset

↓

Reset Circuit

↓

Boot Pins Checked

↓

Boot Mode Selected

↓

Vector Table Read

↓

Stack Pointer Loaded

↓

Reset Handler Executed

↓

SystemInit()

↓

Data Initialization

↓

BSS Initialization

↓

C Library Initialization

↓

main()

# Step 1 : Power-On Reset (POR)

When power is applied,

Reset circuit keeps CPU in reset until

Voltage becomes stable.

Purpose

- Stable startup

- Prevent incorrect execution

# Step 2 : Reset Signal

Sources

- Power-On Reset

- External Reset Pin

- Software Reset

- Watchdog Reset

All cause execution from the reset vector.

# Step 3 : Boot Mode Selection

STM32 checks

BOOT0

BOOT1 (on some devices)

Possible Boot Modes

| Boot Mode | Boot Memory |
| --- | --- |
| User Flash | User Program |
| System Memory | STM32 Bootloader |
| SRAM | RAM Boot |

# Step 4 : Vector Table

The CPU reads the Vector Table, located at the beginning of the selected memory.

The first two entries are critical:

| Address | Content |
| --- | --- |
| 0x00000000 | Initial Stack Pointer (MSP) |
| 0x00000004 | Reset Handler Address |

The remaining entries contain addresses of interrupt service routines.

# Step 5 : Initial Stack Pointer (MSP)

The processor loads the Main Stack Pointer (MSP) from the first word of the vector table.

This initializes the stack before any function calls occur.

# Step 6 : Reset Handler

The processor jumps to the Reset_Handler function.

Responsibilities:

- Initialize system clocks

- Configure memory

- Copy initialized data

- Clear BSS

- Call SystemInit()

- Call C runtime initialization

- Transfer control to main()

# Step 7 : Startup File

The startup file (commonly startup_stm32xxxx.s) is written in assembly language.

It contains:

- Vector table

- Reset handler

- Default interrupt handlers

- Stack definition

- Heap definition (optional)

# Step 8 : SystemInit()

SystemInit() is provided by STM32 CMSIS.

Tasks:

- Configure Flash latency

- Enable FPU (if available)

- Configure PLL

- Configure clock tree

- Initialize system clock

# Step 9 : Data Initialization (.data Section)

The .data section contains initialized global and static variables.

Example:

int count = 10;

Before main():

- Initial values are stored in Flash.

- The Reset Handler copies them from Flash to SRAM.

Flow:

Flash (.data initial values)

│

▼

SRAM (.data runtime variables)

# Step 10 : BSS Initialization (.bss Section)

The .bss section contains uninitialized global and static variables.

Example:

int counter;

Before main():

- The Reset Handler sets all .bss variables to zero.

Flow:

SRAM (.bss)

↓

Filled with Zero

# Why Data and BSS Initialization are Important?

Without initialization:

- Variables contain random values.

- Program behavior becomes unpredictable.

- Software may crash or produce incorrect results.

# Step 11 : C Runtime Initialization

The C runtime:

- Initializes the standard library.

- Executes global constructors (for C++).

- Prepares the runtime environment.

# Step 12 : Execution of main()

Finally:

Reset_Handler

↓

SystemInit()

↓

Initialize .data

↓

Clear .bss

↓

C Runtime

↓

main()

The user application begins execution.

# Complete Boot Flow Diagram

Power ON

↓

Power-On Reset

↓

BOOT0/BOOT1 Check

↓

Select Boot Memory

↓

Read Vector Table

↓

Load MSP

↓

Jump to Reset_Handler

↓

SystemInit()

↓

Copy .data (Flash → SRAM)

↓

Clear .bss (Zero Initialize)

↓

C Runtime Initialization

↓

main()

↓

while(1)

↓

Application Executes

# Advantages of Proper Boot Sequence

- Reliable startup

- Correct variable initialization

- Stable clock configuration

- Proper interrupt setup

- Predictable application execution

- Improved system reliability

# Summary

- SysTick is a 24-bit core timer used for periodic interrupts, delays, RTOS scheduling, and time measurement.

- Configuring SysTick involves selecting the clock source, calculating the reload value, enabling the interrupt, and implementing SysTick_Handler().

- The STM32 boot sequence starts with a reset event, checks boot mode, loads the stack pointer and reset handler from the vector table, initializes the clock and memory, copies initialized data, clears the .bss section, performs C runtime initialization, and finally calls main().

- Understanding SysTick and the boot process is fundamental for developing reliable real-time embedded applications.

# Important Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| ARM | Advanced RISC Machines |
| MCU | Microcontroller Unit |
| CPU | Central Processing Unit |
| CMSIS | Cortex Microcontroller Software Interface Standard |
| SysTick | System Tick Timer |
| NVIC | Nested Vectored Interrupt Controller |
| ISR | Interrupt Service Routine |
| RTOS | Real-Time Operating System |
| POR | Power-On Reset |
| MSP | Main Stack Pointer |
| PSP | Process Stack Pointer |
| PC | Program Counter |
| SP | Stack Pointer |
| FPU | Floating Point Unit |
| PLL | Phase-Locked Loop |
| HSI | High-Speed Internal Oscillator |
| HSE | High-Speed External Oscillator |
| SRAM | Static Random Access Memory |
| Flash | Non-Volatile Flash Memory |
| ROM | Read-Only Memory |
| RAM | Random Access Memory |
| BSS | Block Started by Symbol (uninitialized global/static data section) |
| .data | Initialized global/static data section |
| .text | Executable program code section |
| .rodata | Read-Only Data section (constants) |
| BOOT0 | Boot Mode Selection Pin |
| BOOT1 | Secondary Boot Selection Pin (available on some STM32 devices) |
| RCC | Reset and Clock Control |
| HAL | Hardware Abstraction Layer |
| API | Application Programming Interface |