# ARM Cortex-M7 Vector Table and Nested Vectored Interrupt Controller (NVIC)

# Learning Outcomes

After completing this topic, students should be able to:

- Explain the significance of the Vector Table in ARM Cortex-M7 processors.

- Describe how interrupt vectors are stored in memory.

- Explain how the Cortex-M7 processor responds to an interrupt.

- Describe the architecture of the Nested Vectored Interrupt Controller (NVIC).

- Explain interrupt priorities, preemption, vector table organization, and interrupt servicing.

- Understand the role of NVIC in real-time embedded systems.

# PART – I : Vector Table in ARM Cortex-M7

# 1. Introduction

One of the most important features of ARM Cortex-M processors is the Vectored Interrupt System.

Unlike traditional processors that use software polling to identify interrupt sources, Cortex-M processors use a Vector Table, allowing the processor to identify and service interrupts rapidly.

The Vector Table provides:

- Fast interrupt response

- Deterministic interrupt handling

- Reduced software overhead

- Efficient real-time operation

# What is a Vector?

A vector is the memory address of an Interrupt Service Routine (ISR) or exception handler.

Instead of searching for the correct interrupt routine, the processor directly jumps to the stored address.

Example:

Interrupt Occurs

↓

Vector Table Entry

↓

Address of ISR

↓

Execute ISR

# What is a Vector Table?

The Vector Table is a table stored in memory that contains the addresses of all exception handlers and interrupt service routines.

Each entry in the table corresponds to a specific exception or interrupt.

# Location of Vector Table

Normally stored at

Address

0x00000000

or mapped to

Flash Memory Base

The vector table can also be relocated using the VTOR (Vector Table Offset Register).

# Why is the Vector Table Important?

The Vector Table allows the processor to:

- Start execution after reset

- Locate interrupt handlers instantly

- Support fast interrupt servicing

- Support nested interrupts

- Improve real-time performance

# Structure of the Vector Table

Address        Contents

0x00000000 → Initial Main Stack Pointer (MSP)

0x00000004 → Reset_Handler

0x00000008 → NMI_Handler

0x0000000C → HardFault_Handler

0x00000010 → MemManage_Handler

0x00000014 → BusFault_Handler

0x00000018 → UsageFault_Handler

...

External Interrupt 0

External Interrupt 1

External Interrupt 2

...

USART Interrupt

SPI Interrupt

ADC Interrupt

Timer Interrupt

# First Entries of the Vector Table

| Entry | Purpose |
| --- | --- |
| Entry 0 | Initial Stack Pointer |
| Entry 1 | Reset Handler |
| Entry 2 | Non-Maskable Interrupt (NMI) |
| Entry 3 | Hard Fault Handler |
| Entry 4 | Memory Management Fault |
| Entry 5 | Bus Fault |
| Entry 6 | Usage Fault |
| Remaining Entries | Peripheral Interrupt Handlers |

# Vector Table Block Diagram

Vector Table

+------------------------------------+

| Initial Stack Pointer              |

+------------------------------------+

| Reset Handler                      |

+------------------------------------+

| NMI Handler                        |

+------------------------------------+

| Hard Fault Handler                 |

+------------------------------------+

| Memory Management Handler          |

+------------------------------------+

| Bus Fault Handler                  |

+------------------------------------+

| Usage Fault Handler                |

+------------------------------------+

| SVCall Handler                     |

+------------------------------------+

| PendSV Handler                     |

+------------------------------------+

| SysTick Handler                    |

+------------------------------------+

| GPIO Interrupt                     |

+------------------------------------+

| UART Interrupt                     |

+------------------------------------+

| SPI Interrupt                      |

+------------------------------------+

| Timer Interrupt                    |

+------------------------------------+

| ADC Interrupt                      |

+------------------------------------+

# Types of Exceptions

ARM Cortex-M7 supports

### System Exceptions

Generated internally

Examples

- Reset

- NMI

- Hard Fault

- MemManage Fault

- Bus Fault

- Usage Fault

- SVCall

- PendSV

- SysTick

### External Interrupts

Generated by peripherals

Examples

- GPIO

- UART

- SPI

- I²C

- ADC

- DAC

- DMA

- Timers

- CAN

- USB

# How Interrupt Vectors are Stored

Each vector table entry stores

32-bit Address

↓

ISR Function Address

Example

Vector Table

Entry

UART

↓

0x08001528

↓

UART_IRQHandler()

When UART interrupt occurs,

Processor immediately jumps to

UART_IRQHandler()

# Vector Table Organization

Flash Memory

↓

Vector Table

↓

Exception Addresses

↓

Peripheral ISR Addresses

Each interrupt has one fixed location.

# Interrupt Response Sequence

Suppose

UART receives data.

Sequence

UART Receives Data

↓

Interrupt Request Generated

↓

NVIC Receives Interrupt

↓

Priority Checked

↓

Current CPU State Saved

↓

Vector Table Read

↓

ISR Address Obtained

↓

Branch to ISR

↓

ISR Executed

↓

Return from Interrupt

↓

Previous Program Continues

# Processor Response to Interrupt

Step 1

Interrupt generated

↓

Step 2

NVIC receives request

↓

Step 3

Priority comparison

↓

Step 4

CPU finishes current instruction

↓

Step 5

Registers automatically pushed

↓

Step 6

Vector table accessed

↓

Step 7

ISR executed

↓

Step 8

Registers restored

↓

Program resumes

# Automatic Context Saving

The Cortex-M7 automatically saves

- R0

- R1

- R2

- R3

- R12

- LR (Link Register)

- PC (Program Counter)

- xPSR (Program Status Register)

This process is called automatic stacking.

# Benefits of Automatic Stacking

- Faster interrupt servicing

- Simplifies ISR development

- Lower software overhead

- Faster context switching

# PART – II : Nested Vectored Interrupt Controller (NVIC)

# Introduction

The Nested Vectored Interrupt Controller (NVIC) is an integrated interrupt controller within the ARM Cortex-M core.

It manages:

- Interrupt requests

- Interrupt priorities

- Interrupt nesting

- Preemption

- Exception handling

The NVIC enables Cortex-M7 to achieve very low interrupt latency.

# NVIC Block Diagram

Peripheral Interrupts

GPIO

UART

SPI

ADC

DMA

Timers

↓

Interrupt Requests

↓

+------------------------------+

|             NVIC             |

|------------------------------|

| Enable Registers             |

| Pending Registers            |

| Active Registers             |

| Priority Registers           |

| Vector Controller            |

+------------------------------+

|

|

CPU Core

|

Vector Table

|

Interrupt Service Routine

# Major Components of NVIC

| Component | Function |
| --- | --- |
| Interrupt Enable Registers | Enable interrupts |
| Interrupt Disable Registers | Disable interrupts |
| Pending Registers | Store pending interrupts |
| Active Registers | Show currently active interrupts |
| Priority Registers | Store interrupt priority |
| Software Trigger Register | Generate software interrupts |

# Interrupt Priority

Every interrupt is assigned a priority.

Example

| Priority Number | Importance |
| --- | --- |
| 0 | Highest |
| 1 | High |
| 2 | Medium |
| 3 | Low |
| 15 | Lowest (implementation-dependent number of priority bits) |

Important: In ARM Cortex-M, a lower numerical priority value indicates a higher interrupt priority.

# Why Priorities are Needed

Consider

UART

Timer

ADC

All generate interrupts simultaneously.

Without priorities,

CPU cannot decide

Which interrupt should execute first.

NVIC solves this problem.

# Priority Example

UART

Priority = 1

Timer

Priority = 3

ADC

Priority = 2

Execution order

UART

↓

ADC

↓

Timer

# Interrupt Nesting

Interrupt nesting means

A higher-priority interrupt can interrupt the execution of a lower-priority ISR.

Example

Timer ISR Running

↓

UART Interrupt Arrives

↓

UART Priority Higher

↓

CPU Suspends Timer ISR

↓

UART ISR Executes

↓

Timer ISR Resumes

# Interrupt Preemption

Preemption means

One interrupt temporarily stops another interrupt.

Condition

Higher priority interrupt arrives.

Example

ADC ISR

↓

UART Interrupt

↓

UART Priority Higher

↓

UART Executes

↓

ADC Continues

# Difference Between Nesting and Preemption

| Feature | Interrupt Nesting | Preemption |
| --- | --- | --- |
| Definition | Multiple ISRs execute in a nested manner | Higher-priority ISR interrupts a lower-priority ISR |
| Requirement | Multiple interrupt levels | Higher priority interrupt |
| Result | ISR stack grows | Current ISR is paused and later resumed |

In Cortex-M systems, preemption is the mechanism that enables interrupt nesting.

# Interrupt States

Interrupts may be

- Inactive

- Pending

- Active

- Active and Pending

# Interrupt Servicing Flow

Peripheral Interrupt

↓

NVIC

↓

Priority Check

↓

Current Instruction Completes

↓

Automatic Register Saving

↓

Vector Table Lookup

↓

ISR Executed

↓

Registers Restored

↓

Return to Previous Program

# Tail Chaining

Suppose

Timer ISR finishes

Immediately another interrupt is pending.

Instead of

Restore Context

↓

Save Context Again

The processor performs

Timer ISR Ends

↓

Next ISR Starts Immediately

Benefits

- Faster execution

- Reduced overhead

- Improved real-time performance

# Late Arrival

Suppose

CPU is entering

Timer ISR

Before execution begins,

UART interrupt arrives

Higher priority

CPU immediately switches to UART ISR.

Benefits

- Lower latency

- Faster response

- Better deterministic behavior

# Priority Grouping

Priority bits can be divided into:

- Preemption Priority – Determines whether one interrupt can interrupt another.

- Subpriority – Determines the servicing order when multiple pending interrupts have the same preemption priority.

This configuration is controlled through the Application Interrupt and Reset Control (AIRCR) register.

# Advantages of NVIC

- Very low interrupt latency

- Hardware priority management

- Supports nested interrupts

- Automatic context saving

- Fast exception handling

- Tail chaining

- Late arrival optimization

- Reduced CPU overhead

- Deterministic real-time behavior

# Practical Example

Suppose an STM32H743 system uses:

- Timer Interrupt – Priority 3

- ADC Interrupt – Priority 2

- UART Interrupt – Priority 1

- Emergency GPIO Interrupt – Priority 0

If all interrupts occur nearly simultaneously, the NVIC services them in this order:

- Emergency GPIO Interrupt

- UART Interrupt

- ADC Interrupt

- Timer Interrupt

If the Timer ISR is running and the Emergency GPIO interrupt occurs, the Timer ISR is preempted immediately. After the Emergency ISR completes, the Timer ISR resumes from the point where it was interrupted.

# Summary

- The Vector Table stores the addresses of exception handlers and interrupt service routines, enabling the Cortex-M7 to locate handlers quickly.

- The first vector table entry contains the Main Stack Pointer (MSP), and the second entry contains the Reset Handler.

- When an interrupt occurs, the processor automatically saves the execution context, reads the ISR address from the vector table, executes the ISR, restores the context, and resumes the interrupted program.

- The NVIC manages interrupt enabling, prioritization, pending status, and nesting.

- Lower numerical priority values correspond to higher interrupt priorities.

- Features such as preemption, tail chaining, and late arrival significantly reduce interrupt latency and make the Cortex-M7 highly suitable for real-time embedded systems.

# Important Abbreviations

| Abbreviation | Full Form |
| --- | --- |
| ARM | Advanced RISC Machines |
| MCU | Microcontroller Unit |
| CPU | Central Processing Unit |
| NVIC | Nested Vectored Interrupt Controller |
| ISR | Interrupt Service Routine |
| MSP | Main Stack Pointer |
| PSP | Process Stack Pointer |
| VTOR | Vector Table Offset Register |
| PC | Program Counter |
| LR | Link Register |
| SP | Stack Pointer |
| xPSR | Program Status Register |
| NMI | Non-Maskable Interrupt |
| SVC | Supervisor Call |
| PendSV | Pendable Service Call |
| SysTick | System Tick Timer |
| GPIO | General-Purpose Input/Output |
| UART | Universal Asynchronous Receiver/Transmitter |
| SPI | Serial Peripheral Interface |
| I²C | Inter-Integrated Circuit |
| ADC | Analog-to-Digital Converter |
| DAC | Digital-to-Analog Converter |
| DMA | Direct Memory Access |
| CAN | Controller Area Network |
| USB | Universal Serial Bus |
| AIRCR | Application Interrupt and Reset Control Register |
| CMSIS | Cortex Microcontroller Software Interface Standard |
| FPU | Floating Point Unit |
| RTOS | Real-Time Operating System |

## Classroom Discussion Questions

- Why does the ARM Cortex-M7 use a vector table instead of software polling for interrupt handling?

- Explain the complete sequence of events from interrupt generation to the execution of an ISR.

- How does the NVIC determine which interrupt to service first when multiple interrupts occur simultaneously?

- Compare interrupt priority, preemption, and subpriority with suitable examples.

- Explain how tail chaining and late arrival improve the interrupt performance of the Cortex-M7.

- Describe the role of the VTOR register and discuss scenarios where relocating the vector table is beneficial.

- Why is the Cortex-M7's interrupt architecture considered highly suitable for deterministic real-time embedded applications?