# STM32 Memory Organization with Memory Map

# 1. Introduction to STM32 Memory Organization

## Definition

Memory Organization refers to the arrangement of different memory regions inside the STM32 microcontroller. Each memory region is designed for a specific purpose such as storing programs, variables, stack, peripherals, or external devices.

Unlike traditional 8-bit microcontrollers, STM32 (especially STM32H7 with Cortex-M7 core) contains multiple high-speed memories connected through different buses to improve performance.

The Cortex-M7 processor follows the Harvard Architecture, where instructions and data are accessed simultaneously through separate buses.

# Important Abbreviations

| Abbreviation | Full Form | Purpose |
| --- | --- | --- |
| STM32 | STMicroelectronics 32-bit MCU | ARM-based Microcontroller Family |
| MCU | Microcontroller Unit | Embedded processing device |
| CPU | Central Processing Unit | Executes instructions |
| Flash | Flash Memory | Stores program permanently |
| SRAM | Static Random Access Memory | Temporary data storage |
| ITCM | Instruction Tightly Coupled Memory | High-speed instruction memory |
| DTCM | Data Tightly Coupled Memory | High-speed data memory |
| AXI | Advanced eXtensible Interface | High-speed memory bus |
| AHB | Advanced High-performance Bus | Internal bus |
| APB | Advanced Peripheral Bus | Peripheral communication |
| DMA | Direct Memory Access | Transfers data without CPU |
| NVIC | Nested Vector Interrupt Controller | Interrupt management |
| GPIO | General Purpose Input Output | Digital I/O |
| RCC | Reset and Clock Control | Clock generation |
| ADC | Analog to Digital Converter | Converts analog to digital |
| DAC | Digital to Analog Converter | Converts digital to analog |

# 2. STM32H7 Memory Map

The STM32H7 has a 4 GB (32-bit) address space.

STM32H7 MEMORY MAP

Address

FFFFFFFF -------------------------------

System Memory

E0000000 -------------------------------

Cortex-M7 Internal Registers

NVIC

SysTick

SCB

MPU

60000000 -------------------------------

External Memory

(FMC, SDRAM, NOR Flash)

40000000 -------------------------------

Peripheral Registers

GPIO

USART

SPI

I2C

ADC

TIMERS

RCC

DMA

30000000 -------------------------------

AXI SRAM

24000000 -------------------------------

SRAM

20000000 -------------------------------

DTCM RAM

08000000 -------------------------------

FLASH Memory

00000000 -------------------------------

ITCM RAM / Boot Memory

# Overall Memory Layout

| Memory Region | Typical Address | Purpose |
| --- | --- | --- |
| ITCM RAM | 0x00000000 | Fast instruction execution |
| Flash Memory | 0x08000000 | Program storage |
| DTCM RAM | 0x20000000 | Fast data access |
| AXI SRAM | 0x24000000 / 0x30000000 | General-purpose high-speed RAM |
| Peripheral Space | 0x40000000 | Peripheral registers |
| External Memory | 0x60000000 | External RAM/Flash |
| System Control Space | 0xE0000000 | Cortex-M7 internal registers |

# 3. Flash Memory

## Full Form

Flash = Non-Volatile Flash Memory

## Characteristics

- Permanent memory

- Stores application program

- Retains data after power OFF

- Can be erased and reprogrammed

- Executed directly by CPU (Execute-In-Place)

### Typical Address

0x08000000

### Uses

- Application firmware

- Bootloader

- Interrupt Vector Table

- Constant lookup tables

- Calibration data

### Advantages

✔ Non-volatile

✔ Large storage

✔ Low power

✔ Long retention

### Limitations

- Slower than RAM

- Limited erase cycles

- Erase occurs sector-wise

### Example

int main()

{

while(1)

{

HAL_GPIO_TogglePin(GPIOB,GPIO_PIN_0);

}

}

This program is stored in Flash Memory.

# Flash Memory Structure

Flash

----------------------

Interrupt Vector

----------------------

Startup Code

----------------------

Application Program

----------------------

Constant Data

----------------------

Reserved Area

----------------------

# 4. ITCM RAM

## Full Form

ITCM = Instruction Tightly Coupled Memory

## Definition

ITCM is a very high-speed RAM directly connected to the Cortex-M7 instruction bus.

It is mainly used to execute critical instructions without wait states.

### Address

0x00000000

### Features

- Zero wait-state access

- Dedicated instruction bus

- Faster than Flash

- Deterministic execution

- Used for real-time applications

### Used For

- DSP algorithms

- Motor control

- Interrupt Service Routines

- AI inference kernels

- Fast mathematical functions

### Why ITCM?

Flash may require wait states.

ITCM executes instructions immediately.

CPU

│

│

Instruction Bus

│

ITCM

### Example

Critical interrupt routines may be copied into ITCM during startup.

Flash

↓

Copy

↓

ITCM

↓

CPU executes here

### Advantages

- Extremely fast execution

- Predictable timing

- Ideal for real-time systems

### Limitations

- Small size

- Volatile

# 5. DTCM RAM

## Full Form

DTCM = Data Tightly Coupled Memory

## Definition

DTCM is a high-speed RAM directly connected to the CPU data bus.

It stores frequently accessed variables.

### Address

0x20000000

### Used For

- Stack

- Local variables

- Global variables

- RTOS task stacks

- Buffers

### Data Flow

CPU

│

│

Data Bus

│

DTCM RAM

### Advantages

- Zero wait state

- Very fast read/write

- No cache latency

- Deterministic timing

### Applications

Motor control

Digital filters

FFT

Control algorithms

PID controllers

### Example

int sensorData[100];

This array may be placed in DTCM RAM for faster access.

# 6. AXI SRAM

## Full Form

AXI = Advanced eXtensible Interface

## Definition

AXI SRAM is high-capacity RAM connected through the ARM AXI bus.

Unlike ITCM and DTCM, AXI SRAM is shared by CPU and DMA.

### Address

0x24000000

(or additional AXI SRAM regions depending on STM32H7 variant)

### Features

- Large memory

- Cacheable

- Shared with DMA

- High bandwidth

- Suitable for large buffers

### Applications

Image buffers

Ethernet buffers

USB buffers

LCD frame buffer

Audio processing

Machine learning data

### CPU Access

CPU

│

AXI Bus

│

AXI SRAM

│

DMA

### Advantages

- Large storage

- High throughput

- Multi-master access

- Efficient for bulk data

### Limitations

- Slightly slower than ITCM/DTCM due to bus arbitration

- Performance depends on cache configuration

# 7. Peripheral Memory Region

## Definition

Peripheral memory contains the registers used to configure and control on-chip hardware modules.

The CPU accesses peripherals by reading from or writing to memory-mapped registers.

### Address Range

Starts from

0x40000000

### Examples

GPIO

USART

SPI

I2C

ADC

DAC

DMA

TIMERS

RCC

CAN

USB

Ethernet

### Memory-Mapped I/O

Unlike RAM,

writing to a peripheral address changes hardware behavior.

Example:

GPIOB_ODR

0x48000414

Writing

1

turns an LED ON.

### Block Diagram

CPU

↓

Peripheral Bus

↓

GPIO

USART

SPI

ADC

DAC

Timers

DMA

USB

# 8. System Control Space

Starts from

0xE0000000

Contains

- NVIC

- SysTick

- SCB (System Control Block)

- MPU (Memory Protection Unit)

- Debug registers

# 9. Complete STM32H7 Memory Hierarchy

CPU

│

┌──────────────┼───────────────┐

│              │               │

Instruction      Data Bus        AXI Bus

│              │               │

│              │               │

ITCM          DTCM RAM       AXI SRAM

│              │               │

└──────────────┼───────────────┘

│

Flash Memory

│

Peripheral Bus

│

GPIO USART SPI I2C ADC DAC TIMERS DMA

# 10. Comparison of STM32 Memory Regions

| Feature | Flash | ITCM RAM | DTCM RAM | AXI SRAM | Peripheral Memory |
| --- | --- | --- | --- | --- | --- |
| Full Form | Flash Memory | Instruction Tightly Coupled Memory | Data Tightly Coupled Memory | Advanced eXtensible Interface SRAM | Memory-Mapped Peripheral Registers |
| Volatile | No | Yes | Yes | Yes | N/A (Registers) |
| Speed | Medium | Very High | Very High | High | High |
| CPU Access | Yes | Yes | Yes | Yes | Yes |
| DMA Access | Limited | No | No | Yes | Yes |
| Primary Use | Program code | Critical instructions | Fast variables | Large data buffers | Hardware control |
| Typical Address | 0x08000000 | 0x00000000 | 0x20000000 | 0x24000000 | 0x40000000 |
| Example | Firmware | ISRs, DSP routines | Stack, local variables | Ethernet/image buffers | GPIO, UART, Timers |

# 11. Teaching Summary

- Flash Memory stores the application firmware permanently and retains data after power loss.

- ITCM RAM (Instruction Tightly Coupled Memory) provides zero-wait-state execution for time-critical code such as interrupt service routines and DSP algorithms.

- DTCM RAM (Data Tightly Coupled Memory) offers very fast access to frequently used data, including stacks and control variables.

- AXI SRAM (Advanced eXtensible Interface SRAM) provides larger, high-bandwidth memory suitable for DMA transfers, networking, graphics, and large data buffers.

- Peripheral Memory Region contains memory-mapped control registers for all on-chip peripherals such as GPIO, UART, SPI, I²C, ADC, DAC, timers, DMA, USB, and Ethernet.

- System Control Space contains Cortex-M7 core components such as the NVIC (Nested Vector Interrupt Controller), SysTick (System Timer), SCB (System Control Block), and MPU (Memory Protection Unit).