- STM32CubeMX

- STM32CubeIDE

- STM32 Nucleo Development Board

- Major Components Integrated Inside STM32 Microcontroller

- LED Blinking Experiment using CubeMX and CubeIDE

- Startup File and Boot Process

# STM32CubeMX and STM32CubeIDE

# 1. Introduction to STM32 Software Development

Developing applications for STM32 microcontrollers requires software tools that simplify peripheral configuration, code generation, compilation, debugging, and program downloading.

STMicroelectronics provides two primary software tools:

- STM32CubeMX

- STM32CubeIDE

These tools reduce development time and eliminate the need to manually configure peripheral registers.

# Important Abbreviations

| Abbreviation | Full Form | Purpose |
| --- | --- | --- |
| STM32 | STMicroelectronics 32-bit MCU | ARM Cortex-M Microcontroller |
| IDE | Integrated Development Environment | Software development platform |
| MCU | Microcontroller Unit | Embedded controller |
| HAL | Hardware Abstraction Layer | High-level peripheral library |
| LL | Low Layer | Low-level peripheral library |
| GPIO | General Purpose Input Output | Digital I/O pins |
| RCC | Reset and Clock Control | Clock management |
| SWD | Serial Wire Debug | ARM debugging interface |
| JTAG | Joint Test Action Group | Debugging interface |
| CMSIS | Cortex Microcontroller Software Interface Standard | ARM software framework |
| ELF | Executable and Linkable Format | Executable program file |
| HEX | Intel Hex File | Flash programming file |
| BIN | Binary File | Binary executable |

# 2. STM32CubeMX

## Definition

STM32CubeMX is a graphical software tool used to configure STM32 microcontrollers and automatically generate initialization code.

It eliminates the need for manual register configuration.

## Main Functions

- Device selection

- Clock configuration

- GPIO configuration

- Peripheral configuration

- Middleware configuration

- Pin assignment

- Code generation

## CubeMX Workflow

Select MCU

↓

Configure Clock

↓

Configure GPIO

↓

Configure Peripherals

↓

Generate Code

↓

Open in STM32CubeIDE

# Features of STM32CubeMX

### Graphical Interface

Configuration through mouse clicks.

### Automatic Pin Assignment

Avoids pin conflicts.

### Clock Configuration

Automatically calculates PLL settings.

### Peripheral Initialization

Generates initialization functions.

Example

MX_GPIO_Init();

MX_USART3_UART_Init();

MX_ADC1_Init();

### Middleware Support

Supports

- USB

- FreeRTOS

- LWIP

- FATFS

- TouchGFX

### Code Generation

Automatically generates

- main.c

- gpio.c

- usart.c

- adc.c

- tim.c

# Advantages

✔ No manual register programming

✔ Faster development

✔ Error reduction

✔ Easy peripheral configuration

# 3. STM32CubeIDE

## Definition

STM32CubeIDE is the official Integrated Development Environment for STM32.

It combines

- CubeMX

- GCC Compiler

- Debugger

- Programmer

- Editor

into one software package.

# CubeIDE Architecture

Editor

↓

Compiler

↓

Linker

↓

Debugger

↓

Programmer

↓

STM32 MCU

# Features

### Project Management

Creates STM32 projects.

### C/C++ Editor

Supports

- Syntax highlighting

- Auto completion

- Error detection

### Compiler

GNU ARM GCC Compiler.

### Debugger

Supports

- Breakpoints

- Watch variables

- Memory viewer

- Register viewer

### Programmer

Downloads firmware using

- ST-Link

- JTAG

- SWD

### Console

Displays

- Compilation errors

- Build status

# CubeIDE Advantages

- Free software

- Professional IDE

- Integrated CubeMX

- Advanced debugger

- Cross-platform

# 4. STM32 Nucleo Development Board

## Definition

The STM32 Nucleo board is an official evaluation and development board designed by STMicroelectronics for rapid prototyping and learning STM32 microcontrollers.

It integrates the microcontroller, programmer/debugger, clock source, power supply, user interface, and expansion connectors on a single board.

## Objectives of the Nucleo Board

- Learn STM32 programming

- Develop embedded applications

- Interface sensors and actuators

- Prototype IoT systems

- Test peripherals without additional hardware

# Typical STM32 Nucleo Board

+-------------------------------------------------------+

| USB Connector                                         |

|                                                       |

| ST-LINK Debugger                                      |

|                                                       |

| STM32 Microcontroller                                 |

|                                                       |

| User Button                                           |

| Reset Button                                          |

|                                                       |

| User LED                                              |

|                                                       |

| Arduino Connectors                                    |

| Morpho Connectors                                     |

+-------------------------------------------------------+

# Hardware Resources

## 1. STM32 Microcontroller

Example

STM32F401RE

Contains

- ARM Cortex-M4 CPU

- Flash

- SRAM

- Timers

- ADC

- Communication peripherals

## 2. ST-LINK Onboard Debugger

### Purpose

Programs and debugs the microcontroller.

Functions

- Flash programming

- Breakpoints

- Single stepping

- Register viewing

- Memory inspection

## Debug Interfaces

### SWD

Serial Wire Debug

Uses only

- SWDIO

- SWCLK

Preferred for STM32.

### JTAG

Joint Test Action Group

Uses multiple pins.

Less common on newer Nucleo boards.

## User LED

Usually connected to GPIO.

Used for

- LED blinking

- Status indication

Example

PA5 → LED

## User Push Button

Connected to GPIO interrupt.

Used for

- External interrupt experiments

- User input

## Clock Source

Provides

- External crystal (on some boards)

- Internal oscillator

- PLL

## Arduino Connectors

Compatible with Arduino shields.

Examples

- LCD

- Ethernet

- Motor Driver

- Wi-Fi Shield

## Morpho Connectors

Expose nearly all MCU GPIO pins for advanced applications.

# Communication Interfaces

Integrated interfaces include:

| Interface | Application |
| --- | --- |
| UART/USART | PC communication |
| SPI | LCD, sensors |
| I²C | EEPROM, RTC, sensors |
| CAN | Automotive |
| USB | Host/device |
| Ethernet | IoT networking |
| SDIO | SD card |
| ADC | Analog sensors |
| DAC | Signal generation |

# Power Options

The Nucleo board supports multiple power sources:

- USB power (5 V)

- External VIN

- External regulated 3.3 V

- ST-LINK USB power

Voltage regulators provide the required 3.3 V supply to the MCU and peripherals.

# 5. Major Components Integrated Inside the STM32 Microcontroller

- ARM Cortex-M CPU

- Flash Memory

- SRAM

- GPIO Ports

- RCC (Reset and Clock Control)

- Timers

- ADC

- DAC

- DMA

- NVIC

- SysTick Timer

- UART/USART

- SPI

- I²C

- CAN

- USB

- Ethernet (device-dependent)

- RTC

- Watchdog Timers

- CRC Unit

- Power Management Unit

- Boot Configuration Logic

- Debug Support (SWD/JTAG)

# 6. LED Blinking on STM32 Nucleo using STM32CubeMX and STM32CubeIDE

## Objective

Configure a GPIO pin as an output and blink the onboard LED using HAL libraries.

## Step 1: Create a New Project

- Open STM32CubeIDE.

- Select File → New → STM32 Project.

- Choose the appropriate Nucleo board or MCU (e.g., NUCLEO-F401RE).

- Click Next.

- Enter a project name.

- Click Finish.

## Step 2: Configure the GPIO in STM32CubeMX

- Open the Pinout & Configuration tab.

- Select the onboard LED pin (e.g., PA5 on many Nucleo boards).

- Set the pin mode to GPIO_Output.

- Ensure the system clock is configured (default settings are sufficient for this experiment).

- Save the project (Ctrl + S).

- CubeMX automatically generates the initialization code.

## Step 3: Generated Initialization Code

CubeMX creates functions such as:

MX_GPIO_Init();

SystemClock_Config();

These are called automatically from main().

## Step 4: Write the LED Blinking Code

Insert the following code inside the while (1) loop:

while (1)

{

HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5);

HAL_Delay(500);

}

This toggles the LED every 500 ms, producing a blinking effect.

## Step 5: Build (Compile) the Project

- Click the Build button (hammer icon).

- The GCC compiler translates the C source code into machine code.

- If there are no errors, executable files such as .elf, .hex, and .bin are generated.

## Step 6: Connect the Nucleo Board

- Connect the board to the computer using a USB cable.

- The onboard ST-LINK debugger is detected automatically.

## Step 7: Download the Program

- Click Run or Debug.

- The firmware is downloaded through the ST-LINK interface.

- The program is written into the STM32 Flash memory.

## Step 8: Debug the Program

CubeIDE supports:

- Breakpoints

- Step Into

- Step Over

- Variable Watch

- Register View

- Memory View

- Call Stack

These tools help verify program execution and identify errors.

## Step 9: Program Execution

After programming:

- The CPU starts from the Reset Handler.

- The system clock and GPIO are initialized.

- The main() function is executed.

- The LED blinks continuously.

## Flow of LED Blinking

Create Project

↓

Configure GPIO

↓

Generate Code

↓

Write Application Code

↓

Compile Project

↓

Download to STM32

↓

Debug (Optional)

↓

LED Blinks

# 7. Startup File in STM32 Projects

## Definition

The startup file is an assembly language source file (for example, startup_stm32f401xe.s or startup_stm32h743xx.s) that is executed immediately after the microcontroller is reset.

It prepares the hardware and software environment before the C program (main()) begins execution.

# Purpose of the Startup File

The startup file performs the following essential tasks:

- Initializes the Main Stack Pointer (MSP).

- Defines the Interrupt Vector Table.

- Executes the Reset Handler.

- Initializes the .data section by copying initialized variables from Flash to SRAM.

- Clears the .bss section (zero-initializes uninitialized global and static variables).

- Calls the C runtime initialization.

- Invokes the main() function.

# Structure of a Startup File

Interrupt Vector Table

↓

Initial Stack Pointer

↓

Reset Handler

↓

Default Exception Handlers

↓

Peripheral Interrupt Handlers

# Stack Initialization

The first entry in the vector table contains the initial value of the Main Stack Pointer (MSP).

When the MCU resets, the Cortex-M core automatically loads this value into the stack pointer before executing any instructions.

# Interrupt Vector Table

The interrupt vector table is an array of addresses located at the beginning of the program memory.

It contains:

- Initial Stack Pointer

- Reset Handler

- NMI Handler

- Hard Fault Handler

- SysTick Handler

- Peripheral interrupt handlers (USART, TIM, ADC, EXTI, etc.)

When an interrupt occurs, the processor automatically reads the corresponding address from the vector table and jumps to that Interrupt Service Routine (ISR).

# Reset Handler

The Reset Handler is the first executable function after a reset.

Its responsibilities include:

- Initializing memory sections (.data and .bss)

- Calling SystemInit() to configure clocks and low-level hardware

- Invoking the C library initialization

- Calling the main() function

Execution sequence:

Power-On / Reset

↓

Load Stack Pointer

↓

Reset Handler

↓

SystemInit()

↓

Initialize .data and .bss

↓

Call main()

↓

User Application Executes

# Teaching Summary

- STM32CubeMX is a graphical configuration tool that generates initialization code for clocks, GPIOs, and peripherals.

- STM32CubeIDE integrates project creation, code editing, compilation, debugging, and programming in a single environment.

- The STM32 Nucleo board provides an easy-to-use hardware platform with an onboard ST-LINK debugger, user LED, push button, communication interfaces, and flexible power options.

- STM32 microcontrollers integrate numerous peripherals—including Flash, SRAM, GPIO, timers, ADC, DAC, communication interfaces, DMA, and NVIC—on a single chip, reducing system complexity.

- Developing an LED blinking application follows a standard workflow: create project → configure GPIO in CubeMX → generate code → write application → compile → download → debug → execute.

- The startup file is a critical part of every STM32 project. It initializes the stack, defines the interrupt vector table, executes the Reset Handler, prepares memory, and transfers control to the main() function, where the user application begins.